	[image: image1.png]




	[image: image2.jpg]Meicheng






	I NSTRUCTION :

The SB-4144 is a true Matrix Routing Switcher for VGA (RGBHV)/Stereo Audio(AR/AL) signals. It is an 4 individual VGA (RGBHV)/Stereo Audio (AR/AL) inputs with 4 individual RGBHV/Audio outputs. Because it is a matrix router, any input may be routed to any output; or the same input may be routed to all outputs or any combination. It completely eliminates the need to constantly move around audio and video input cables and output category (RJ-45) cables. 

The SB-4144 is useful for Matrix signals from VGA source devices (such as: Personal computer, Set Top Box, and Satellite Receivers, etc.) To VGA destination devices (Such as LCD Monitors, VGA Monitors, Plasma LCD TV, VGA Projectors, etc.). Selection of inputs is made via the front panel push buttons or an Infrared Remote Control unit or RS 232 control by a computer.

APPLICATION :


	4x VGA-Audio source devices to 
4x VGA-Audio Display
[image: image3.jpg])

I
BE@mE

SB-4144





[image: image4.jpg]





	 INSTALLING :
 
CONTROL PORTS
 
1. 
IR REMOTE - IR Remote Controller.
2.
3.

RS 232 Interface - RS 232 interface system.
Manual controlled by front panel button.

 
 
 
 
 
 
 
 
 
 FEATURES :
 
01. Supports 4x inputs VGA+Audio to 4x VGA-Audio outputs Matrix Switcher 
02. Input signals support Via VGA (RGBHV), Stereo Audio (AR/AL) 
03. Output Signals support VGA and Stereo Audio to VGA Monitor display 
04. Higher Video Bandwidth 325MB each path R,G,B signals. 
05. Supported HD high definition resolutions XGA, SXGA, UXGA, WSXGA, WUXGA 
06. RS232 Serial interface via ethernet control is optional 
07. Compatible with all VGA Video Monitor devices, Plasma display and Projectors 
08. Supported RS232 serial interface protocol commands list 
09. Control PC RS232 Drive compatible with win-95/98/2000/xp 
10. Various User Interface controls: 
[image: image5.png]


Attached Window based control software for Desktop or NB control by RS232 port 
[image: image6.png]


Manual controlled by front panel button 
[image: image7.png]


IR remote control 
11. Support desktop with ear mount and 19 inch rack mountable type panel 
12. Power supply DC12 Volt, Universal Switch 100~230V AC, 50/60Hz
 
 
 SPECIFICATIONS :


	Type of Switcher : 
4 in To 4 out, VGA (RGBHV)-Audio Matrix Switcher 
Input ports: 
4x VGA (RGBHV) via HD-15p VGA connector 
 
4x Stereo Audio (AR/AL), Via 3.5mm Ear Phone Jack 
Output ports: 
4x VGA (RGBHV) via HD-15p VGA connector 
 
4x Stereo Audio (AR/AL), Via 3.5mm Ear Phone Jack 
Video Bandwidth : 
325MHz (-3db), 200mVp-p 
Video Supported : 
Higher resolution formats HD 1920x1200 (WUXGA) 
Audio Supported : 
Stereo Audio 
Low all hostile crosstalk :
-83 db@5MHz 
Controls :
IR remote, Select buttons on the front panel & RS232 
PC RS232 Control :
RS232 interface serial via DRV on a PC
Gain control :
60MHz 0.1 db gain flatness 
Chassis Material :
Metal thin=1mm
Safety Approvals :
CE, FCC, RoHS (2002/95/EC). 
Dimensions (LWH) :
19”x 7.87”  x 1.73”(482mm x 200mm x 44mm) 
Power Supply :
DC12V / 1A~2A (consumption 800mA Max)
Use Universal Switch Type 50/60Hz,100~230 VAC
Shipping Weight :
2.25 Kg / 3.75 lb





®





SB-4144  4 x 4 VGA -AUDIO Matrix Switcher





Meicheng





www.meicheng.com.tw








Meicheng®    MEI CHENG AUDIO VIDEO CO., LTD.�
�
Address:13F, No. 2, Jian 8th Rd., Jhonghe City, Taipei County 23511, TAIWAN


Tel: +886(2) 8228 0311,       Fax: +886(2) 8228 0319


Website : www.meicheng.com.tw         Emai l:mei.cheng@msa.hinet.net�
�









