| Meicheng ® www.meicheng.com.tw CSC-1600HD/ Video Scaler |  |
| --- | --- |
| Applications: |  |
| CSC-1600HD is a high performance intelligent video scaler for converting any standard of video to, among others, non-interlaced high resolution HDTV 720P picture or SXGA (1280 x 1024) pc resolution. With a variety of output pixel rate, you can always choose a suitable resolution to match the native resolution of your display TV/Monitor such that the final displayed picture is free of distortion and at highest quality. |  |
| Features: |  |
| - Video inputs are de-interlaced and scaled up to output resolution as below: PC output: 640x480, 800x600 (SVGA), 1024x768 (XGA), 1280x1024 (SXGA), 1280x768(WXGA),1365x768 HDTV output: 852x480p, 852x576p, 1280x720p @16:9, 1920x1080i @16:9 - Automatically accommodates worldwide input video systems of NTSC3.58, NTSC4.43, PAL, PAL M,PAL N and SECAM. - High performance adaptive 4H Comb filter Y/C separator with adjustable vertical peaking. - Per-pixel motion compensated de-interlacing algorithms to produce artifact-free progressive scan video signal. - Built in Adaptive film mode 3:2 pull-down provides clear and crisp de-interlacing of video originating from 24 fps film, such as DVD movies. - Frame rate up conversion from 50 Hz up to 75Hz for PAL input, 60 Hz up to 85Hz for NTSC input. - Vertical temporal filter (VT) removes jaggy and other de-interlacing artifacts from normal video. - OSD menu with adjustable control on Color, Sharpness, Brightness, Contrast and Tint . - Built-in DCTI/DLTI circuit for color/luminance transient improvement. - The PC loop-through allows for easy change between video and PC source. - Compatible with all TVs or projectors with YPbPr or RGBHV input. - Last memory for all adjustments. - RS-232 interface allows remote control from PC. - IR remote control - Dimensions - Desktop 1.75"H x 17.2"W x 7"D (44 x 438 x 178mm) - Dimensions - Desktop 1.75"H x 19"W x 7"D (44 x 482 x 178mm) |  |

Meicheng® MEI CHENG AUDIO VIDEO CO., LTD.

Address:13F, No. 2, Jian 8th Rd., Jhonghe City, Taipei County 23511, TAIWAN Tel: +886(2) 8228 0311, Fax: +886(2) 8228 0319 Website: www.meicheng.com.tw Email:mei.cheng@msa.hinet.net